DiffBot

ROS2 + Gazebo Simulation with Real Arduino/L298N Hardware Bridge

Project Documentation

Arduino Uno · L298N Motor Driver · 2× IMR 18650 Li-ion · ROS2 Humble · Gazebo · rosbridge

# Table of Contents

# 1. Project Overview

This project implements a keyboard-controlled 4-wheel differential-drive robot car, built and validated in two parallel forms simultaneously: a physics-accurate Gazebo simulation running under ROS2, and a real Arduino Uno + L298N + 2× 18650 Li-ion hardware rig. Both forms share the exact same command interface — the ROS2 topic /cmd_vel — so the same keyboard input drives the simulated robot on screen and the physical car on the floor at the same time.

The system was built and debugged end-to-end in a single session: from an empty workspace, through Gazebo simulation, Arduino firmware, a hardware wiring fault, a ROS2–serial bridge, and finally a browser-based teleoperation controller with proper press-and-hold / release-to-stop behavior.

## 1.1 Goals

- Drive a robot car using keyboard input.

- Have that same input control both a Gazebo simulation and a real Arduino-based car.

- Visualize the robot's motion/heading feedback live, in simulation.

- Make control feel natural: hold a key to move, release to stop immediately, for both simulated and real robot.

## 1.2 Hardware Used

| Component | Detail |
| --- | --- |
| Microcontroller | Arduino Uno |
| Motor driver | L298N dual H-bridge module |
| Motors | 4× DC gear motors, 2 per side (left pair, right pair) |
| Battery | 2× IMR 18650 Li-ion, wired in series (~7.4–8.4V) |
| Speed control | None — ENA/ENB jumpered to 5V on the L298N board itself; on/off (bang-bang) control only, no PWM |
| Host PC | Ubuntu 22.04.5 LTS, ROS2 Humble, Gazebo Classic |

# 2. System Architecture

The design principle behind this project is that simulation and hardware are two independent 'bodies' driven by one shared 'brain' — the ROS2 /cmd_vel topic. Nothing about the simulation needs to know the real car exists, and nothing about the real car needs to know the simulation exists; a bridge node on each side simply translates the same command stream into the appropriate output.

## 2.1 High-level Data Flow

Browser Teleop Page (HTML/JS)

keydown -> publish Twist @10Hz while held

keyup   -> publish zero-Twist immediately

|

v   (WebSocket, via rosbridge_server)

/cmd_vel  (geometry_msgs/Twist)

|

-------------------------------

|                             |

v                             v

Gazebo diff_drive plugin   serial_bridge_node.py

(simulated physics)        (translates Twist -> serial

|                        commands for Arduino)

v                             |

/odom (ground truth,             v

from sim physics)         Arduino Uno (L298N)

-> real motors spin

## 2.2 Why Two Separate Control Paths?

Gazebo's diff_drive plugin natively accepts a Twist and produces smooth simulated motion with true physics-based odometry feedback on /odom. The real hardware has no such built-in interface — it only understands a custom serial protocol sent to the Arduino over USB. The serial_bridge_node.py node exists purely to translate ROS2's standard Twist message into that hardware-specific protocol, so that from the user's perspective (and from any other ROS2 node's perspective), the real robot behaves like a normal ROS2-controlled robot.

## 2.3 Component List

| Component | Role |
| --- | --- |
| diffbot_sim (ROS2 package) | URDF robot description, Gazebo world, launch files for simulation |
| gazebo_ros diff_drive plugin | Converts /cmd_vel into simulated wheel motion + publishes /odom |
| diffbot_final.ino (Arduino firmware) | Receives serial commands, drives L298N in on/off mode, safety watchdog |
| serial_bridge_node_onoff.py | ROS2 node: /cmd_vel -> serial commands to Arduino; also publishes a dead-reckoning /odom_estimate |
| rosbridge_server | WebSocket bridge exposing ROS2 topics to browser JavaScript |
| diffbot_teleop.html | Browser-based controller with true keydown/keyup (hold-to-move, release-to-stop) |

# 3. Hardware Setup

## 3.1 Final Confirmed Wiring

This is the wiring configuration that was tested and confirmed working. ENA/ENB are NOT connected to the Arduino — they are jumpered directly to the L298N board's own 5V pin, permanently enabling both motor channels. This means the car has no variable speed control: each side is simply ON (forward), ON (reverse), or OFF.

L298N IN1  ->  Arduino D7   (left side, direction pin A)

L298N IN2  ->  Arduino D8   (left side, direction pin B)

L298N IN3  ->  Arduino D9   (right side, direction pin A)

L298N IN4  ->  Arduino D10  (right side, direction pin B)

L298N ENA  ->  L298N 5V pin (jumper wire, board-to-board only)

L298N ENB  ->  L298N 5V pin (jumper wire, board-to-board only)

L298N GND  ->  Arduino GND  (common ground -- required)

L298N 12V  ->  Battery pack + (2x 18650 in series)

L298N GND (power terminal) -> Battery pack -

## 3.2 Hardware Issue Encountered & Resolution

During integration, the motors initially did not spin at all despite correct code, confirmed power (L298N status LED lit), confirmed common ground, and confirmed working motors (tested directly against the battery). The root cause was twofold:

- ENA/ENB were left floating (not tied to a logic HIGH anywhere), which meant the L298N's internal H-bridge outputs were disabled at the hardware level regardless of what IN1–IN4 were set to. Fix: jumper wires from ENA and ENB to the board's own 5V pin.

- A loose/undisturbed dupont connector was also suspected as a secondary contributor after a firmware re-upload appeared to change behavior; a full firm reseat of every connector (IN pins, motor terminals, battery terminals, ground jumper) resolved the remaining inconsistency.

NOTE: The ENA/ENB jumper is not optional. It is a hardware requirement of the L298N chip itself — without a HIGH signal there, the H-bridge will not switch, no matter what the firmware does.

## 3.3 Known Hardware Limitations

- No PWM speed control — motors are full-speed or off only, per side.

- No wheel encoders — there is no real position/heading feedback from the physical robot. The /odom_estimate topic published by serial_bridge_node.py is a dead-reckoning estimate only (integrated from commanded velocity over time) and will drift, particularly during turns.

# 4. Software Setup

## 4.1 Environment

| Item | Version / Detail |
| --- | --- |
| OS | Ubuntu 22.04.5 LTS (Jammy) |
| ROS2 distribution | Humble |
| Simulator | Gazebo Classic (via gazebo_ros) |
| Build tool | colcon |
| Serial library | pyserial |
| Browser bridge | ros-humble-rosbridge-suite |

## 4.2 One-time Installation

Most packages were already present in this environment. If starting from a fresh Ubuntu 22.04 + ROS2 Humble install, run:

sudo apt update

sudo apt install ros-humble-gazebo-ros-pkgs ros-humble-gazebo-ros \

ros-humble-xacro ros-humble-robot-state-publisher \

ros-humble-joint-state-publisher ros-humble-joint-state-publisher-gui \

ros-humble-teleop-twist-keyboard ros-humble-rviz2 xterm \

ros-humble-rosbridge-suite

pip install pyserial --break-system-packages

# allow serial port access without sudo each time

sudo usermod -a -G dialout $USER   # then log out/in once

## 4.3 Workspace Build

cd ~/diffbot_ws

colcon build --symlink-install

source install/setup.bash

To avoid re-sourcing manually every session, this was added to ~/.bashrc:

echo 'source /opt/ros/humble/setup.bash' >> ~/.bashrc

echo 'source ~/diffbot_ws/install/setup.bash' >> ~/.bashrc

## 4.4 Project Directory Structure

diffbot_ws/

├── README.md

├── SESSION_NOTES.md

├── src/diffbot_sim/                    (ROS2 package)

│   ├── package.xml

│   ├── CMakeLists.txt

│   ├── urdf/diffbot.urdf.xacro          4-wheel robot description

│   ├── launch/gazebo_sim.launch.py       spawns world + robot

│   ├── launch/teleop.launch.py           (superseded by browser teleop)

│   ├── worlds/empty_arena.world

│   └── config/, rviz/, meshes/           (placeholders)

├── arduino/diffbot_l298n/

│   └── diffbot_final.ino                 CURRENT working firmware

├── serial_bridge/

│   └── serial_bridge_node_onoff.py       CURRENT working bridge node

└── teleop_web/

└── diffbot_teleop.html               browser hold-to-move controller

# 5. Gazebo Simulation

## 5.1 Robot Description

The robot is modeled in diffbot.urdf.xacro as a 4-wheel differential-drive chassis: a rectangular box chassis with 4 cylindrical wheels (continuous joints). Physically, the left-side pair and right-side pair are ganged together electrically on the real hardware, so in simulation the gazebo_ros diff_drive plugin controls the front_left_wheel_joint and front_right_wheel_joint directly, while the rear wheels mimic the front wheels' motion visually.

## 5.2 Key Plugin Parameters (diffbot.urdf.xacro)

| Parameter | Purpose |
| --- | --- |
| left_joint / right_joint | Which wheel joints receive the differential-drive velocity commands |
| wheel_separation / wheel_diameter | Physical dimensions used for the Twist -> wheel velocity conversion |
| command_topic | Set to cmd_vel — matches the shared control topic used everywhere else |
| odometry_topic / publish_odom_tf | Publishes ground-truth simulated position/heading feedback |
| max_wheel_acceleration | Raised from 5.0 to 20.0 so the simulated robot reaches speed quickly instead of feeling sluggish |

## 5.3 World

empty_arena.world defines a simple ground plane, a sun light source, and two static colored boxes as visual landmarks, so rotation and translation are easy to perceive on screen.

## 5.4 Launching the Simulation

cd ~/diffbot_ws

ros2 launch diffbot_sim gazebo_sim.launch.py

This launch file starts Gazebo with the arena world, runs robot_state_publisher (publishing TF from the URDF), spawns the diffbot entity into the simulation, and opens RViz2 alongside it.

NOTE: A bug was found and fixed here during development: robot_state_publisher failed to parse the xacro-generated robot_description because it was passed as a raw Command substitution instead of an explicit string. Fix: wrap it in ParameterValue(Command(...), value_type=str).

# 6. Arduino Firmware

## 6.1 Serial Protocol

A simple text-based protocol was designed so it is easy to test manually via the Arduino IDE's Serial Monitor before any ROS2 code is involved:

| Direction | Format | Meaning |
| --- | --- | --- |
| PC -> Arduino | L:<val>,R:<val>\n | val > 0 = forward, val < 0 = reverse, val = 0 = stop, per side. Magnitude is ignored (no PWM on this hardware). |
| Arduino -> PC | OK:<left>,<right>\n | Acknowledgement echo, sent once per accepted command. |

## 6.2 Safety Watchdog

The firmware automatically stops both motors if no new command is received within 500ms. This prevents a runaway car if the USB/serial connection drops unexpectedly. This is also why continuous command publishing (rather than a single one-off message) is required from the ROS2 side — addressed in Section 8.

## 6.3 Final Firmware (diffbot_final.ino)

const uint8_t IN1 = 7;   // left  motors, direction A

const uint8_t IN2 = 8;   // left  motors, direction B

const uint8_t IN3 = 9;   // right motors, direction A

const uint8_t IN4 = 10;  // right motors, direction B

const unsigned long CMD_TIMEOUT_MS = 500;

void driveSide(uint8_t pinA, uint8_t pinB, int cmd) {

if (cmd > 0)      { digitalWrite(pinA, HIGH); digitalWrite(pinB, LOW); }

else if (cmd < 0) { digitalWrite(pinA, LOW);  digitalWrite(pinB, HIGH); }

else              { digitalWrite(pinA, LOW);  digitalWrite(pinB, LOW); }

}

// loop(): reads 'L:x,R:y' lines from Serial, applies motor state,

// replies 'OK:x,y', and auto-stops if no command for 500ms.

// Full source is included in the project repository.

The complete, uploaded source file is diffbot_final.ino in the arduino/diffbot_l298n/ directory of the project repository.

# 7. ROS2 ↔ Arduino Serial Bridge

## 7.1 Purpose

serial_bridge_node_onoff.py is a ROS2 node that subscribes to /cmd_vel, converts the incoming Twist (linear.x, angular.z) into a bang-bang left/right command matching the real hardware's on/off-only capability, and writes it to the Arduino over USB serial. It also performs simple dead-reckoning to estimate the real robot's position, published on /odom_estimate, clearly distinguished from Gazebo's ground-truth /odom.

## 7.2 Twist-to-Motor Conversion Logic

left_val  = v - (w * wheel_separation / 2)

right_val = v + (w * wheel_separation / 2)

# thresholded to bang-bang (no PWM available):

left_cmd  = +100 if left_val  >  deadzone

= -100 if left_val  < -deadzone

=   0 otherwise

right_cmd = (same logic for right_val)

## 7.3 Running the Bridge

# 1. Identify the Arduino's serial port (varies by USB-serial chip):

ls /dev/ttyACM* /dev/ttyUSB* 2>/dev/null

# this hardware enumerates as /dev/ttyUSB0 (CH340-style chip)

# 2. Ensure Arduino IDE's Serial Monitor is CLOSED

#    (only one program may hold the serial port at a time)

# 3. If 'Device or resource busy':

sudo fuser -k /dev/ttyUSB0

# 4. Start the bridge:

cd ~/diffbot_ws

python3 serial_bridge/serial_bridge_node_onoff.py \

--ros-args -p serial_port:=/dev/ttyUSB0

NOTE: /odom_estimate is a dead-reckoning estimate only. This hardware has no wheel encoders, so there is no ground-truth position feedback from the real robot — this is an accepted, documented limitation, not a bug.

# 8. Teleoperation Control

## 8.1 Evolution of the Control Method

Three approaches were tried in order, each solving a limitation of the previous one:

| Method | Behavior | Limitation |
| --- | --- | --- |
| teleop_twist_keyboard (default) | Publishes one Twist per keypress | Real car's 500ms safety watchdog stops it ~1s after a single keypress, since no repeat commands are sent |
| teleop_twist_keyboard --repeat-rate 10.0 | Republishes the last Twist continuously at 10Hz | Solves the real-car timeout, but terminal input cannot detect key-release — the robot keeps moving until a different key (k) is explicitly pressed |
| Browser-based teleop (diffbot_teleop.html) | Publishes at 10Hz while a key is held (keydown), and immediately publishes zero-Twist the instant the key is released (keyup) | None — this is the final, correct solution for true press-and-hold control |

## 8.2 Why a Browser Page Was Needed

Standard terminal keyboard input in Linux has no concept of a key-release event — a terminal only ever reports characters typed, not physical key states. Detecting 'key is currently held down' versus 'key was pressed once' requires an interface that tracks raw hardware key events, which browsers provide natively via the keydown and keyup JavaScript events. rosbridge_server exposes ROS2 topics over a WebSocket specifically so that browser JavaScript (via the roslibjs library) can publish and subscribe to them directly.

## 8.3 rosbridge Setup

sudo apt install -y ros-humble-rosbridge-suite

# launch (opens a WebSocket server on port 9090):

ros2 launch rosbridge_server rosbridge_websocket_launch.xml

## 8.4 Browser Controller (diffbot_teleop.html)

Key behaviors implemented in the page:

- Connects to ws://localhost:9090 automatically on page load, shows a live connection status indicator.

- keydown on i / , / j / l begins publishing Twist messages at 10Hz for as long as the key remains held.

- keyup immediately publishes a single zero-Twist message — no delay, no dependence on the repeat timer.

- On-screen buttons mirror the same behavior via mouse/touch events, for use without keyboard focus.

- If the browser tab loses focus while a key is held (e.g. Alt-Tab), all motion is force-stopped as a safety measure.

- Both /cmd_vel consumers (Gazebo's diff_drive plugin and serial_bridge_node) receive identical commands, so simulated and real motion are synchronized.

## 8.5 Speed Tuning

The real car (on/off, full-speed motors) moves noticeably faster than Gazebo's initial default simulated speed. Rather than precisely measuring the real car's speed, the simulated speed was tuned upward by feel to visually match:

// diffbot_teleop.html

const LINEAR_SPEED = 1.2;   // m/s while forward/back held (was 0.3)

const ANGULAR_SPEED = 3.0;  // rad/s while turning held (was 1.0)

Additionally, the Gazebo diff_drive plugin's acceleration cap was raised so the simulated robot reaches its target speed quickly rather than ramping up slowly:

<!-- diffbot.urdf.xacro -->

<max_wheel_torque>20</max_wheel_torque>

<max_wheel_acceleration>20.0</max_wheel_acceleration>

# 9. Full Run Sequence (Every Session)

This is the complete, current, correct sequence to bring the whole system online from a cold boot. Four terminals/windows are used.

## Terminal 1 — Gazebo Simulation

cd ~/diffbot_ws

ros2 launch diffbot_sim gazebo_sim.launch.py

Wait ~20-30 seconds. Confirm the Gazebo window opens with the 4-wheel robot visible, and RViz2 opens alongside it.

## Terminal 2 — rosbridge WebSocket Server

ros2 launch rosbridge_server rosbridge_websocket_launch.xml

Wait for confirmation the WebSocket server has started (port 9090).

## Terminal 3 — Real Arduino Serial Bridge

# plug in Arduino via USB, connect the battery to the L298N

# CLOSE Arduino IDE Serial Monitor if it is open

ls /dev/ttyACM* /dev/ttyUSB* 2>/dev/null    # confirm port

cd ~/diffbot_ws

python3 serial_bridge/serial_bridge_node_onoff.py \

--ros-args -p serial_port:=/dev/ttyUSB0

Wait for the line: Connected to Arduino on /dev/ttyUSB0

## Step 4 — Open the Browser Controller

xdg-open ~/diffbot_ws/teleop_web/diffbot_teleop.html

Confirm the status dot turns green ('Connected to rosbridge'). Hold i / , / j / l to drive — both the Gazebo simulation and the real car respond together, and both stop the instant a key is released.

## 9.1 One-Line Combined Reference

For quick copy-paste, here is the full sequence as one block (run each line in its own terminal, in order):

# Terminal 1

cd ~/diffbot_ws && ros2 launch diffbot_sim gazebo_sim.launch.py

# Terminal 2

ros2 launch rosbridge_server rosbridge_websocket_launch.xml

# Terminal 3  (adjust serial_port if not /dev/ttyUSB0)

cd ~/diffbot_ws && python3 serial_bridge/serial_bridge_node_onoff.py --ros-args -p serial_port:=/dev/ttyUSB0

# Then open in a browser:

xdg-open ~/diffbot_ws/teleop_web/diffbot_teleop.html

# 10. Troubleshooting Reference

| Symptom | Cause | Fix |
| --- | --- | --- |
| robot_description as yaml parse error on launch | Command substitution passed without an explicit string type | Wrap in ParameterValue(Command(...), value_type=str) in gazebo_sim.launch.py |
| FileNotFoundError: xterm | xterm not installed | sudo apt install -y xterm |
| Motors don't spin, LED on L298N is lit, code uploads and acks fine | ENA/ENB floating -- H-bridge outputs disabled at hardware level | Jumper ENA and ENB directly to the L298N's own 5V pin |
| Real car worked once, then stopped after a re-upload | Loose dupont connector disturbed during handling | Firmly reseat every connector: IN pins, motor terminals, battery terminals, ground jumper |
| Could not open serial port: No such file or directory | Wrong port name assumed | Check with ls /dev/ttyACM* /dev/ttyUSB* -- this board uses /dev/ttyUSB0, not ttyACM0 |
| Could not open serial port: Device or resource busy | Another program (often Arduino IDE Serial Monitor) holds the port | Close Serial Monitor; if needed, sudo fuser -k /dev/ttyUSB0 |
| Real car stops ~1s after a single keypress | Arduino's 500ms safety watchdog with no repeated commands | Use continuous publishing: --repeat-rate 10.0, or the browser teleop page |
| Robot won't stop when a key is released (terminal teleop) | Terminal input has no key-release event | Use the browser-based teleop page (diffbot_teleop.html), which uses real keydown/keyup |
| Gazebo motion feels slow compared to the real car | Default simulated speed too low relative to full-speed real motors | Increase LINEAR_SPEED / ANGULAR_SPEED in diffbot_teleop.html and max_wheel_acceleration in the URDF |

# 11. Future Work / Upgrade Path

- Add wheel encoders (e.g. IR slot sensors on one left + one right wheel) to replace dead-reckoning with true measured odometry, eliminating drift.

- Add PWM speed control by wiring ENA/ENB to Arduino PWM-capable pins, enabling variable speed instead of on/off only.

- Build a saved RViz2 configuration (rviz/diffbot.rviz) to visualize TF, odometry path trail, and both /odom and /odom_estimate side by side for comparison.

- Calibrate estimated_speed in serial_bridge_node_onoff.py against a measured real-world distance/time test, to make /odom_estimate a closer approximation.

- Consider an IMU (e.g. MPU6050) for heading feedback, which is significantly more reliable than wheel-based dead reckoning for rotation specifically.

— End of document —